OASIS are introducing some changes to the current ULTRA model.

This model will be called ULTRA 2.0.

The main purpose of the change is to improve serviceability, standardize components across other models, reduction of water fittings & improve assembly & quality.

This change is planned to come into effect in June / July 2025

ULTRA 2.0 will have the same Cooling, Carbonation & Heating system as the current ULTRA.

The table below outlines the main changes and benefits.

| Current ULTRA | ULTRA 2.0 | Comments |
| --- | --- | --- |
| Single Shelf | 2 shelves | Less clutter. Easier access of serviceable components. |
| Difficult Solenoid access | Ease of Solenoid access | 4 x Solenoids mounted together on shelf. |
| Pressurized Cooling Tank | Non pressurized CT | Gravity fed CT. Eliminates erratic flow from Cold dispense. |
| Ultra Cooling Tank | Modified Ultra Cooling Tank | Additional vent outlet added to top of Cooling Tank. |
| Difficult to Air Test on line | Easier to Air Test | 100% Water System Air Test. |
| 3 Dispense points | Single Dispense point | Elimination of potential leak points + Sanitization benefits. |
| Drip Tray grille – 3 coloured rings | Drip Tray grille – 1 ring | Use 1 Blue ring …under dispense point. |
| Co2 Mixer- Can’t be emptied | Mixer easily emptied | Easily emptied by using combination of dispense buttons. |
| Expansion Tank | No expansion Tank | Not necessary. Can now vent back into reservoir. |
| Flow Regulator | No Flow Regulator | Not necessary. Flow restrictor added to Co2 outlet. |
| Numerous JG Elbow fittings. | Many elbows removed | Replaced by corner former clips. Reduction of leak points. |
| Difficult Reservoir removal | Easily removed Res. | Disconnect fittings and twist reservoir from shelf slots. |
| AQ&Q Pump | Kalix style pump | Standardization of parts |
| Numerous lead wires | Reduction of wires | Introduction of new 12 pin Wiring Loom. |
| Difficult Front Panel removal | Easily removed F Panel | Disconnect single 12 pin connector & feed thru Front Panel. |
| PCB | Different PCB | To accommodate new 12 Pin Wiring Loom. |
| Reservoir | Different Reservoir | To accommodate new Cooling Tank venting system. |
| HT replacement | Easier to replace HT | Secure to shelf by rotating in slots and secure using snap clip. |

Additional Tech Bulletin with specific part numbers will be issued closer to release date.